AEOLUS AVIONICS ROUND 2 – TASK 2

AARUSH VASHISTH

2026B7PS0569H

RC Transmitter and receiver:

The Radio Control (RC) system consists of two components: the transmitter (TX) and the receiver (RX). The transmitter is the hand-held controller that is used for sending commands by the user to the on-board receiver which is a small electronic circuit mounted inside the drone (or any other vehicle for that matter). The transmitter reads the physical inputs of the user, translates them into a radio signal and sends it to the receiver (wirelessly) in the drone which then translates the radio signal back into a physical action.

Telemetry Module:

In the above mentioned RC system, communication is one-way, i.e., the user sends a signal via the transmitter to the drone but the drone doesn’t send any data back to the user. Telemetry module changes this and makes the communication two-way. It’s a device that collects data from the sensors and sends it wirelessly to the transmitter in the controller (examples of data transmitted – altitude, battery voltage, coords).

Video Transmitter (VTX):

A VTX is a device on your drone that transmits the live video feed from the onboard camera to your screen or FPV goggles. Analog systems have lower latency when compared to digital ones but they also have lower resolution.

The models I selected:

RC system - Radiomaster TX16S (Transmitter) paired with a Radiomaster RP1 ExpressLRS (ELRS) 2.4GHz Receiver. ELRS is a modern long range protocol which uses a digital serial connection (CRSF) which is fast and uses only 4 wires unlike PWM which needs a separate wire for every channel. PPM multiplexes onto one wire but suffers from latency and jitter.

Telemetry Module - Holybro SiK Telemetry Radio V3 433MHz. 433MHz penetrates obstacles and travels further than 2.4GHz, making it highly reliable for continuous data streaming over the ocean.

VTX – Walksnail Avatar HD Pro Kit. This is a digital VTX and my justification is that it can provide 720p/1080p HD feed which is better than the low res feed from an analog VTX.

Connections:

https://docs.holybro.com/autopilot/pixhawk-6c-mini/pixhawk-6c-mini-ports

RC – Wire the receiver’s TX and RX pads to the RX and TX pins on TELEM 2 port (UART 5). Wire the RX’s 5V and GND pads to the VCC (+5V) and GND pins on the TELEM 2 port.

Telemetry Module – Utilises all 6 ports of TELEM 1 and hence powered from TELEM’s VCC (+5V) and grounded via the GND.

VTX – Wire its TX and RX lines to UART8_RX and UART8_TX pins on the GPS2 port.

Protocols:

ELRS RC Link: Communicates over CRSF (Crossfire Serial Protocol) via 2.4GHz LoRa. It boasts exceptional ranges easily exceeding 10 km to 30+ km with line-of-sight, ensuring you won't lose control over the ocean.

Holybro SiK Telemetry: Communicates using the MAVLink protocol. The 433MHz frequency typically provides a reliable range of 1 km to 3 km.

Walksnail VTX: Uses a proprietary Digital HD Video Protocol. With a clear line of sight, it pushes video up to 4 km, though flying low over water can reduce this slightly due to signal reflection.

Communication Diagram: